# PCB Manufacturing ERP/MES Implementation Plan

Generated on 2025-10-11 13:32

Scope: End-to-end process from RFQ (Gerber+BOM intake) through CAM, PCB fabrication, SMT/assembly, testing, shipping. Separate login per phase and a quality-control gate after every phase.

# 1. End-to-End Workflow

Customer RFQ via portal → Upload Gerber & BOM → automatic DFM/DRC → Review Dashboard for corrections/approval.

Sales quotation → customer acceptance & payment/PO → Sales Order.

Production Approval (capacity + materials) → auto-generate Work Order with traveler & fixed revision.

CAM release: NC drill, phototools/legend, panelization, testpoints, IPC class/tolerances, fixtures.

PCB fabrication routing per traveler with QC after each step and NCR/rework loop on failure.

SMT/Assembly/Box-build: screen print → P&P → reflow → touch-up → ICT → FT → firmware flash → wiring → labelling → pack → despatch.

# 2. ERP/MES Modules

CRM/RFQ & Customer Portal

DFM/CAM control & e-sign release

Item/BOM/AVL with revisions

Costing & Quoting

Production Planning & Scheduling

MES/Shop-floor (station dashboards, traveler scan)

Quality (inspection plans, FAI/PDIR, NCR/CAPA, SPC)

Inventory/Procurement/Stores with lot traceability

Maintenance (PM schedules)

Test Data Capture (ICT/Flying-probe/FT)

Shipping/Logistics & labeling

Finance (invoices, receipts)

Document Control (SOPs, ECNs)

Analytics (OTD, FPY, yields, cost)

# 3. Roles & Logins

Total distinct station/role logins planned: 53

## 3.1 Business / Control Roles

Sales Executive

Sales Manager / Approver

Costing / Estimation

CAM / DFM Engineer

Production Planner / Scheduler

Materials / Stores

Purchasing

QA Manager

Finance / Accounts

Shipping & Logistics Coordinator

System Admin / IT

## 3.2 PCB Fabrication Stations

Count: 27

| Step / Station | QC Gate after Step |
| --- | --- |
| Review Dashboard / Document control | Yes |
| Job-card preparation | Yes |
| Sheet cutting | Yes |
| CNC drilling | Yes |
| Create NC drill (CAM) / Tooling | Yes |
| Create photo films & legend (CAM) | Yes |
| PTH + flash plating | Yes |
| Photo-image transfer | Yes |
| Photo-image inspection (QC) | Yes |
| Alkaline etching | Yes |
| Dry-film inspection (QC) | Yes |
| Resist strip | Yes |
| Dry-film stripping | Yes |
| Pattern plating | Yes |
| Tin stripping | Yes |
| Etching inspection (QC) | Yes |
| Solder mask printing & exposing | Yes |
| Solder mask inspection (QC) | Yes |
| HAL | Yes |
| HAL inspection (QC) | Yes |
| Legend printing | Yes |
| CNC routing | Yes |
| V scoring | Yes |
| Flying-probe testing (QC) | Yes |
| Final QC (PDIR) | Yes |
| Packing | Yes |
| Despatch | Yes |

## 3.3 SMT/Assembly/Wiring Stations

Count: 15

| Station | QC Gate after Step |
| --- | --- |
| Solder-paste screen printing | Yes |
| Pick & place | Yes |
| Reflow oven | Yes |
| Manual soldering / touch-up | Yes |
| ICT testing | Yes |
| Functional testing | Yes |
| Software / firmware flashing | Yes |
| Wire cutting | Yes |
| Wire crimping | Yes |
| Wire connector insertion | Yes |
| Wire load testing | Yes |
| Wire electrical connection testing | Yes |
| Labelling | Yes |
| Packing | Yes |
| Despatch | Yes |

# 4. Quality & Control Points

Barcode traveler scan and operator login at each station; machine ID and start/stop time recorded.

Mandatory inspection checklist with photos/measurements; SPC where applicable.

Automatic hold on QC fail; NCR created; rework routing enforced; re-QC before proceeding.

Engineering revision lock; only CAM can change revision; WO bound to specific revision.

Materials lot/feeder traceability; MSD baking logs.

Firmware images under ENG control; checksum recorded during flashing.

# 5. Dashboards

Review: incoming RFQs, DFM status, corrections outstanding

Planning: capacity, bottlenecks, WO Gantt, material shortages

Shop-floor: per-station queue, WIP aging, rework queue

Quality: FPY, yields by step, NCR cycle time, Pareto

Commercial: quote hit-rate, OTD, AR/AP

Compliance: calibration due, ESD audits, operator training

# 6. Implementation Phases

Phase 0 – Master data & RBAC (items, work centers, routing library).

Phase 1 – RFQ→Quote→SO + file intake & review dashboard.

Phase 2 – CAM release & Materials/MRP.

Phase 3 – PCB Fabrication MES (all stations + QC).

Phase 4 – SMT/Assembly/Test/Wiring MES.

Phase 5 – Quality (NCR/CAPA), Shipping, Finance connections.

Phase 6 – Analytics, SPC, maintenance, advanced test data capture.

# 7. Verification: Flow Steps ↔ Logins ↔ QC

Mapped directly from the PCB XPRESS flow chart; Stage-QC required after every step.

| Flow Step | Mapped Login / Role | Explicit QC Node in Chart | Stage QC after Step (Requirement) | Verification |
| --- | --- | --- | --- | --- |
| Input Data from Customer | Customer Portal (External) | No | Yes | PASS |
| CAM from Customer Data | CAM Engineer (Intake) | No | Yes | PASS |
| Create NC Drill File | CAM – NC Drill | No | Yes | PASS |
| Create File for Photo Film‑Image & Legend | CAM – Phototools/Legend | No | Yes | PASS |
| Job-Card Preparation | Production Control / Job Card | No | Yes | PASS |
| Sheet Cutting | Sheet Cutting Operator | No | Yes | PASS |
| CNC Drilling | CNC Drilling Operator | No | Yes | PASS |
| PTH + Flash Plating | PTH Line Operator | No | Yes | PASS |
| Photo‑Image Transfer | Photo Imaging Operator | No | Yes | PASS |
| Photo‑Image Inspection | QA – Photo Imaging Inspection | Yes | Yes | PASS |
| Alkaline Etching | Etching Line Operator | No | Yes | PASS |
| Tin Stripping | Tin Strip Operator | No | Yes | PASS |
| Etching Inspection | QA – Etch Inspection | Yes | Yes | PASS |
| Dry Film Inspection | QA – Dry Film Inspection | Yes | Yes | PASS |
| Strip (Resist Strip) | Resist Strip Operator | No | Yes | PASS |
| Dry Film Stripping | Dry Film Strip Operator | No | Yes | PASS |
| Pattern Plating | Pattern Plating Operator | No | Yes | PASS |
| Solder Mask Printing & Exposing | Solder Mask Operator | No | Yes | PASS |
| Solder Mask Inspection | QA – Solder Mask Inspection | Yes | Yes | PASS |
| HAL | HAL Operator | No | Yes | PASS |
| HAL Inspection | QA – HAL Inspection | Yes | Yes | PASS |
| Legend Printing | Legend Printer Operator | No | Yes | PASS |
| CNC Routing | CNC Routing Operator | No | Yes | PASS |
| V Scoring | V‑Score Operator | No | Yes | PASS |
| Flying Probe Testing | Test – Flying Probe | Yes | Yes | PASS |
| FINAL QC (PDIR) | QA – Final Inspection / PDIR | Yes | Yes | PASS |
| PACKING | Packing Operator | No | Yes | PASS |
| DESPATCH | Dispatch / Shipping | No | Yes | PASS |

# Appendix: Source Flow & Notes

Double-side PCB Manufacturing Flow Chart

Handwritten QA/Stage inspection notes

Handwritten process steps (CNC, PTH, Photo, Plating, Etching, SM, AOI)